# Benchmarking and Evaluation Methodology

To assess the performance of the Raspberry Pi 5 for onboard bird detection, a standardized set of benchmarking tests was conducted across all team members. Due to differences in individual software implementations, each member retained their own functioning YOLO-based detection code; however, all evaluations were performed under a common experimental setup to ensure comparability. The objective of these evaluations was to quantify detection accuracy, responsiveness, robustness, and operational stability under controlled and repeatable conditions that approximate the sensing constraints of a small unmanned aircraft system.

## Experimental Test Setup

In lieu of a drone-mounted platform, a bench-level approximation was used wherein each Raspberry Pi’s camera was directed toward a secondary display (monitor or laptop). This display presented a predetermined sequence of still images and video clips of birds and non-birds. This served as a reproducible stimulus source for the image-recognition algorithms running on each Pi, regardless of local implementation differences. The content of the slideshow was jointly defined by the team and constituted the common “ground truth” input set for all evaluations.

Each item in the slideshow—whether still image or video—was assigned a unique identifier. The team also annotated whether each item contained a bird, as well as any relevant scene characteristics (e.g., cluttered background, low light, small or distant target). Establishing this ground truth allowed direct comparison between the detector’s output and expected classifications.

#### Table 1 — Slideshow Ground Truth Reference

| ID | Type | Bird Present (Y/N) | Notes (Scene Description) |
| --- | --- | --- | --- |
| Slide 1 | Image | Y | Single bird, close range, clear lighting |
| Slide 2 | Image | Y | Single bird, medium distance |
| Slide 3 | Image | Y | Single bird, distant |
| Slide 4 | Image | Y | Multiple birds |
| Slide 5 | Image | Y | Partially occluded bird (branches, leaves) |
| Slide 6 | Image | Y | Cluttered background (trees) |
| Slide 7 | Image | Y | Bird against plain sky |
| Slide 8 | Image | Y | Low-light bird |
| Slide 9 | Image | Y | Bird slightly blurred (motion) |
| Slide 10 | Image | Y | Bird camouflaged |
| Slide 11 | Image | N | Clear sky |
| Slide 12 | Image | N | Cloudy sky |
| Slide 13 | Image | N | Dense trees |
| Slide 14 | Image | N | Urban scene |
| Slide 15 | Image | N | Shoreline |
| Slide 16 | Image | N | Horizon |
| Slide 17 | Image | N | Kite |
| Slide 18 | Image | N | Human figure |
| Slide 19 | Image | N | Airplane |
| Slide 20 | Image | N | Drone |
| Slide 21 | Video | Y | Bird in slow, steady flight across the sky |
| Slide 22 | Video | Y | Flock of birds with varied movement |
| Slide 23 | Video | Y | Bird obscured by forest flying away |
| Slide 24 | Video | N | Moving clouds |
| Slide 25 | Video | N | Moving through trees |

## Test Procedures

Four categories of evaluations were carried out. These were designed to characterize (1) accuracy under controlled conditions, (2) behavior when presented with moving stimuli, (3) sensitivity to viewpoint and display variations, and (4) operational stability during extended operation.

### 1. Still-Image Accuracy Evaluation

The first test quantified the detector’s performance when presented with static imagery. Images 1-20 from the slideshow was shown on the monitor for five seconds while the Pi’s camera captured the screen and the YOLO model produced classification outputs. For each slide, detections were marked as:

- True Positive (TP): a bird was present and detected.

- False Negative (FN): a bird was present but not detected.

- False Positive (FP): a bird was detected when none was present.

- True Negative (TN): no bird was present and none was detected.

Precision and recall were computed from the aggregate counts.

Precision = TP / (TP + FP), Recall = TP / (TP + FN)

#### Table 2 — Still Image Accuracy Summary

| Tester Name | Platform | Total Slides | TP | FN | FP | TN | Precision | Recall |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 2. Video-Based Detection Performance

The second test examined performance when the target object was moving. This more closely resembles the operational scenario of an sUAS encountering birds in flight. Team members played slides 21-25 with each video clip full-screen while the detector processed the live camera feed directed at the monitor. Each video set to a duration of approximately 10 seconds. Evaluators noted:

- Whether detections were consistently produced during segments containing birds.

- Whether false positives occurred during bird-absent segments.

- The qualitative smoothness of inference (e.g., smooth, acceptable, choppy).

- Approximate/average frame rates when available.

#### Table 3 — Video Detection Summary

| Tester Name | Platform | Video ID | Bird Present (Y/N) | Reliable Detection (Y/N) | False Positives (Y/N) | Smoothness | Approx. FPS |
| --- | --- | --- | --- | --- | --- | --- | --- |

### 3. Robustness Under Varying Presentation Conditions

To approximate less ideal sensing environments, the third test varied the presentation conditions. Slides 3 (single bird, distant), 5 (partially occluded bird), 8 (low-light bird), 11 (clear sky), 13 (dense trees),19 (airplane), and 20 (drone) were played for a 5 second duration. Three representative degradations were tested:

- Increased viewing distance, reducing apparent bird size.

- Reduced display brightness, approximating low-light scenes.

- Oblique camera angles, simulating off-axis viewpoints.

For each condition, a smaller subset of images (containing both bird and non-bird cases) was presented. Results were again logged as TP, FN, FP, and TN for each condition.

#### Table 4 — Robustness Evaluation Summary

| Tester Name | Platform | Condition | Slides Tested | TP | FN | FP | TN | Precision | Recall | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 4. Short-Term Stability and Responsiveness

The fourth evaluation assessed the resilience of the system during continuous operation. A looping sequence of slides was presented for a period of approximately ten minutes. Testers monitored:

- Whether the detector crashed or terminated unexpectedly,

- Whether performance degraded noticeably over time,

- Whether detections remained responsive and timely throughout the test.

#### Table 5 — Stability and Responsiveness

| Tester Name | Platform | Duration (min) | Crashes (Y/N) | Slowdowns (Y/N) | Consistent Detections (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- | --- |

## Relation to Performance Objectives

These benchmarking procedures map directly to the quantitative objectives of this study. Real-time performance and detection latency are reflected in the frame-rate data and qualitative smoothness assessments. Detection accuracy is derived from the aggregate precision and recall values across both static and dynamic tests. Robustness is evaluated through systematic variation of display conditions, and system reliability is demonstrated through continuous-operation testing. Collectively, these evaluations provide a comprehensive assessment of the Raspberry Pi platform’s suitability for onboard bird-detection tasks in support of collision-avoidance systems for small unmanned aircraft.