# Benchmarking Methodology for Bird-Detection Performance Evaluation

This section presents the standardized benchmarking procedure used by all team members to evaluate the performance of the YOLO-based bird detection system running on the Raspberry Pi 5, with and without an attached AI accelerator. Although individual team members used slightly different code implementations due to setup variations, the following methodology ensured consistent and comparable evaluation results across platforms.

The evaluation was performed using a physical camera pointed at a secondary display device (monitor or laptop). The secondary display cycled through a predefined set of bird and non-bird images and video clips. This method enabled reproducible testing without the need for drone-mounted hardware or outdoor data collection.

## 1. Benchmark Materials

### 1.1 Slideshow Dataset

A shared slideshow deck consisting of still images and short video clips was used as the common test input for all evaluations. Each item in the slideshow was assigned a unique identifier (Slide 1, Slide 2, Video A, etc.) and labeled with its ground-truth classification:

- Whether a bird is present (Yes/No)

- Scene characteristics (e.g., close-range bird, small/distant bird, cluttered background, sky-only, low-light conditions)

The team generated an agreed-upon ground truth table (Table 1) to ensure consistent reference across all tests.

## 2. Test Procedures

Each test was performed on two platforms when available:

- Raspberry Pi 5 (baseline CPU execution)

- Raspberry Pi 5 with AI accelerator attached (Hailo AI Hat)

All teammates used their own functioning YOLO inference script without modification, ensuring the procedure was transferable regardless of differences in local codebases.

### 2.1 Test 1 — Still Image Accuracy Evaluation

Objective:
Assess detection accuracy for static images under controlled conditions.

Procedure:

- Launch the YOLO-based detection application.

- Position the Pi’s camera such that it captures the entire monitor display area.

- Display still image slides 1-20, slide duration is set to 5 seconds.

- Record whether the system produced a bird detection for each slide.

- Compare model output with the ground truth classification.

- For each slide, categorize results into:

  - True Positive (TP)

  - False Negative (FN)

  - False Positive (FP)

  - True Negative (TN)

Summary Metrics:
Precision = TP / (TP + FP)
Recall = TP / (TP + FN)

### 2.2 Test 2 — Video-Based Detection Performance

Objective:
Evaluate performance on dynamic scenes, including birds in motion and varying backgrounds.

Procedure:

- Play slides 21-25, a full-screen video clip on the secondary monitor. Each video set to a duration of approximately 10 seconds.

- Observe the detector’s ability to correctly identify birds while the video is in motion.

- Record:

  - Whether bird detections are consistent throughout segments where a bird is present.

  - Presence of false detections during non-bird segments.

  - Apparent smoothness of video processing (qualitative assessment).

  - Approximate/average frame rate if the application displays FPS.

### 2.3 Test 3 — Robustness Evaluation

Objective:
Evaluate detection continuity under varied presentation conditions.

Conditions Tested:

- Increased monitor distance (birds appear smaller)

- Reduced screen brightness (low-light simulation)

- Off-axis viewing angle (camera positioned to simulate non-frontal perspectives)

Procedure:

- Adjust a single presentation condition.

- Display slides 3 (single bird, distant), 5 (partially occluded bird), 8 (low-light bird), 11 (clear sky), 13 (dense trees),19 (airplane), and 20 (drone) were played for a 5 second duration.

- Record TP/FN/FP/TN classifications under each condition.

- Compute precision and recall.

### 2.4 Test 4 — Short-Term Stability and Responsiveness

Objective:
Verify consistent operation and responsiveness over an extended period.

Procedure:

- Initiate a continuous slideshow of all slides for 10 minutes.

- Observe performance for:

  - Crashes or application termination

  - Noticeable slowdowns over time

  - Reduced detection frequency or responsiveness

- Capture qualitative notes and any reported FPS behavior.

## 3. Results Tables

The following tables are used by each team member to record observed performance results. These tables are intended for direct inclusion in the report appendix or results section.

### Table 1 — Slideshow Ground Truth Reference

| ID | Type | Bird Present (Y/N) | Notes (Scene Description) |
| --- | --- | --- | --- |

### Table 2 — Still Image Accuracy Summary

| Tester Name | Platform | Total Slides | TP | FN | FP | TN | Precision | Recall |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Table 3 — Video Detection Summary

| Tester Name | Platform | Video ID | Bird Present (Y/N) | Reliable Detection (Y/N) | False Positives (Y/N) | Smoothness | Approx. FPS |
| --- | --- | --- | --- | --- | --- | --- | --- |

### Table 4 — Robustness Evaluation Summary

| Tester Name | Platform | Condition | Slides Tested | TP | FN | FP | TN | Precision | Recall | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Table 5 — Stability and Responsiveness

| Tester Name | Platform | Duration (min) | Crashes (Y/N) | Slowdowns (Y/N) | Consistent Detections (Y/N) | Notes |
| --- | --- | --- | --- | --- | --- | --- |

## 4. Alignment With Project Objectives

This evaluation methodology directly supports the project’s measurable objectives:

- Real-time operation and latency
Evaluated through smoothness and FPS observations during video playback and stability testing.

- Detection accuracy
Quantified through precision and recall from the still-image and robustness evaluations.

- Robustness to environmental variation
Assessed through manipulations of distance, brightness, and viewing angle.

- Reliability under continuous operation
Demonstrated during the 5–10 minute stability test.

This standardized procedure enables consistent, reproducible comparison across different Raspberry Pi configurations and ensures compatibility with each team member’s independently functioning system.