The Raspberry Pi AI HAT+ used in this study incorporates the Hailo-8L neural processing unit (NPU), a specialized edge-AI accelerator designed to augment the general-purpose CPU of the Raspberry Pi 5. NPUs are optimized for the tensor and matrix operations that dominate modern neural-network inference, enabling significantly higher throughput and lower latency than CPU-only execution. In this project, inference on the Raspberry Pi 5’s ARM Cortex-A76 cores is constrained by their general-purpose, sequential execution model and limited efficiency for large-scale tensor operations. Offloading neural-network workloads to the Hailo-8L therefore enables real-time object detection in scenarios where CPU-based processing would otherwise introduce reduced frame rates or increased latency.

The Hailo-8L is designed by Hailo Technologies and fabricated by TSMC using a 16-nanometer process node, a mature manufacturing technology well suited for low-power embedded systems.  The accelerator provides up to 13 trillion operations per second (13 TOPS) of neural-network inference performance while typically consuming approximately 1.5 watts of power.  Integration with the Raspberry Pi 5 is achieved over a PCIe Gen 3 interface, allowing efficient data transfer between the host and the accelerator.  In the context of this research, the inclusion of the Hailo-8L improves per-frame inference time and effective frame rate, reducing missed detections and increasing reliability for real-time bird detection using an onboard camera. This CPU-NPU combination provides a compact and power-efficient platform suitable for bench-level collision-avoidance research relevant to small unmanned aircraft systems.

Raspberry Pi AI Hat+, Raspberry Pi Ltd, Oct. 2024, datasheets.raspberrypi.com/ai-hat-plus/raspberry-pi-ai-hat-plus-product-brief.pdf.

Gaming, Corsair. “CPU vs GPU VS NPU: What’s the Difference?” CORSAIR, 4 Mar. 2025, www.corsair.com/us/en/explorer/diy-builder/power-supply-units/cpu-vs-gpu-vs-npu-whats-the-difference/?srsltid=AfmBOooXjdJUqLlGbPk_qghF6D_8V6YcjYPYPD-feldGEHTd7wwWxyxb&utm_source=chatgpt.com.

Raspberry Pi 5, Raspberry Pi Ltd, Dec. 2025, pip.raspberrypi.com/documents/RP-008348-DS-3-raspberry-pi-5-product-brief.pdf.

Gaming, Corsair. “CPU vs GPU VS NPU: What’s the Difference?” CORSAIR, 4 Mar. 2025, www.corsair.com/us/en/explorer/diy-builder/power-supply-units/cpu-vs-gpu-vs-npu-whats-the-difference/?srsltid=AfmBOooXjdJUqLlGbPk_qghF6D_8V6YcjYPYPD-feldGEHTd7wwWxyxb&utm_source=chatgpt.com.

Maxfield, Max. “Are These the Top-Performing Edge AI Processors?” EEJournal, Techfocus Media inc, 1 Sept. 2023, www.eejournal.com/article/are-these-the-top-performing-edge-ai-processors/?utm_source=chatgpt.com.

“Technology Node.” WikiChip, WikiChip, 5 Oct. 2025, en.wikichip.org/wiki/technology_node.

“8L Entry-Level AI Accelerator Solutions.” Hailo, Hailo Technologies Ltd, 19 Oct. 2025, hailo.ai/products/ai-accelerators/hailo-8l-ai-accelerator-for-ai-light-applications/#hailo8l-overview.

Raspberry Pi AI Hat+, Raspberry Pi Ltd, Oct. 2024, datasheets.raspberrypi.com/ai-hat-plus/raspberry-pi-ai-hat-plus-product-brief.pdf.

Raspberry Pi AI Hat+, Raspberry Pi Ltd, Oct. 2024, datasheets.raspberrypi.com/ai-hat-plus/raspberry-pi-ai-hat-plus-product-brief.pdf.

Gaming, Corsair. “CPU vs GPU VS NPU: What’s the Difference?” CORSAIR, 4 Mar. 2025, www.corsair.com/us/en/explorer/diy-builder/power-supply-units/cpu-vs-gpu-vs-npu-whats-the-difference/?srsltid=AfmBOooXjdJUqLlGbPk_qghF6D_8V6YcjYPYPD-feldGEHTd7wwWxyxb&utm_source=chatgpt.com.